|  | Tesla, Inc. Service Bulletin |  |  | # Replace Front Seat Assemblies |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SB-24-13-004 December 4, 2024 |  |  |  |  |  |  |
| Classification |  |  |  | Section/Group | Mobile Service | Configuration |
| Recall Bulletin |  |  |  | 13 - Seats | Cannot Perform | All |
| Model |  | Model Year |  | Country/Region | Build Location |  |
| Model Y |  | 2024 |  | North America | Giga Texas |  |
| The model(s) and model year(s) listed are a general approximation of the affected VIN list. Refer to the VIN/Bulletin Tracker or Customer/Vehicle profile to determine applicability of this bulletin for a particular vehicle. |  |  |  |  |  |  |

Recall Bulletin: Recall Bulletins are mandatory service procedures that, unless they can be completed via a software update that a customer installs through the normal course of ownership, must be carried out by Tesla-certified Service Centers. Recall work performed by uncertified technicians could lead to unsafe conditions or voided warranty provisions.

# Condition

On Certain Model Y vehicles, the weld which attaches the LH and/or RH front seat recliner mechanism to the seat back may not have sufficient penetration to assure the durability and integrity of the seat assembly, which may increase the risk of injury in the event of a collision.

# Correction

Use the Vehicle Identification Number (VIN) lookup tool to identify the affected front seat assemblies, and then replace the affected front seat assemblies with seats of weld quality meeting specification.

| Correction Description | Correction | Time |
| --- | --- | --- |
| Replace LH Front Seat Assembly; SB-24-13-004 | S012413004 | 0.36 |
| Replace RH Front Seat Assembly; SB-24-13-004 | S022413004 | 0.42 |
| Replace LH and RH Front Seat Assemblies; SB-24-13-004 | S032413004 | 0.66 |

|  | Part Number | Description |  | Quantity |
| --- | --- | --- | --- | --- |
| Parts Required | 1458818-00-B | BOLT PF M10-1 5X36 109 G0310 B SMAT - TRACK TO PODIUM BOLT |  | 4 or 8 |
| If Necessary | 6654323-T1-B 6654323-T2-B | LH Front Seat Assembly 1R LHD LH SEAT 2AB TX BLACK Or 1R LHD LH SEAT 2AB TX WHITE |  | 1 1 |
|  | 6654325-T1-C 6654325-T2-C | RH Front Seat Assembly 1R LHD RH SEAT 1AB TX BLACK Or 1R LHD RH SEAT 1AB TX WHITE |  | 1 1 |
|  | These part numbers were current at the time of publication. Use the revisions listed or later, unless otherwise specified in the Parts Catalog. |  |  |  |
| Special Tools | 1133787-00-A 1100482-00-A |  | KEY, SEAT BELT RELEASE TP45 TORX PLUS Bit Sockets |  |

# Procedure

Double-click the yellow box in the VIN lookup tool below, enter the full VIN, and then press Enter to determine the front seat assemblies that need to be replaced.

Replace the front seat assemblies that were identified in step 1 (refer to Service Manual procedure 13040102).