|  | Tesla, Inc. Service Bulletin |  |  | # Replace Drive Inverter(s) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SB-24-40-009 November 27, 2024 |  |  | R1 |  |  |  |
| Classification |  |  |  | Section/Group | Mobile Service | Configuration |
| Recall Bulletin |  |  |  | 40 - Rear Drive Unit, 39 - Front Drive Unit | Cannot Perform | All |
| Model |  | Model Year |  | Country/Region | Build Location |  |
| Cybertruck |  | 2024 |  | United States | Giga Texas |  |
| The model(s) and model year(s) listed are a general approximation of the affected VIN list. Refer to the VIN/Bulletin Tracker or Customer/Vehicle profile to determine applicability of this bulletin for a particular vehicle. |  |  |  |  |  |  |

Recall Bulletin: Recall Bulletins are mandatory service procedures that, unless they can be completed via a software update that a customer installs through the normal course of ownership, must be carried out by Tesla-certified Service Centers. Recall work performed by uncertified technicians could lead to unsafe conditions or voided warranty provisions.

This Service Document supersedes SB-24-40-009, dated November 14, 2024. This new revision, R1, updates the VIN lookup tool output for improved compatibility, and changes an FRT. Each content change is marked by a vertical line in the left margin. Discard the previous version and replace it with this one.

# Condition

Certain Cybertruck vehicles are equipped with a drive inverter(s) that contains an internal component that does not meet Tesla specifications. These inverters may fail and cause loss of propulsion.

# Correction

Use the Vehicle Identification Number (VIN) lookup tool to determine which drive inverter is affected, and then replace the drive inverter(s) and/or drive unit as determined by the “REPLACEMENT(S) NEEDED” field.

NOTE: Although this bulletin affects drive inverters, if the VIN lookup tool instructs to replace the drive unit, replace the drive unit as instructed. The replacement drive unit contains the proper drive inverter.

NOTE: If this bulletin is applicable to the vehicle, perform the correction without any need for inspection.

| Correction Description | Correction | Time |
| --- | --- | --- |
| Dual Motor; Replace Front Drive Inverter | S012440009 | 2.46 |
| Dual Motor; Replace Rear Drive Inverter | S022440009 | 3.72 |
| Dual Motor; Replace Front And Rear Drive Inverters | S032440009 | 5.22 |
| Tri-Motor; Replace Front Drive Unit | S042440009 | 2.64 |
| Tri-Motor; Replace LH Rear Drive Inverter | S052440009 | 2.16 |
| Tri-Motor; Replace RH Rear Drive Inverter | S062440009 | 2.16 |
| Tri-Motor; Replace LH And RH Rear Drive Inverters | S072440009 | 2.88 |
| Tri-Motor; Replace Front Drive Unit And LH Rear Drive Inverter | S082440009 | 4.32 |
| Tri-Motor; Replace Front Drive Unit And RH Rear Drive Inverter | S092440009 | 4.50 |
| Tri-Motor; Replace Front Drive Unit And LH And RH Rear Drive Inverters | S102440009 | 5.16 |
| Dual Motor; Replace Rear Drive Unit And Front Drive Inverter | S112440009 | 5.40 |
| Dual Motor; Replace Rear Drive Unit | S122440009 | 3.18 |

|  | Part Number | Description | Quantity |
| --- | --- | --- | --- |
| Parts Required | 1920000-T1-K 1111543-00-A 1907760-00-A 1802305-4A-B 1096344-00-D 1802305-4A-B 1096344-00-D 1802305-4A-B 1096344-00-D 1920000-TX-H | For Tri-Motor Front Drive Unit Replacement FRONT DRIVE UNIT - TRI MOTOR NUT,HF,M12,STL[10],ZNFL,PTP.4 KEPS,FW,HL,M16,[10],G1009,PTP For Tri-Motor LH and/or RH Rear Drive Inverter Replacement ASSEMBLY - DRIVE INVERTER - 4DU LABEL - COVER - ACCESS - LUG - PHASE For Dual Motor Rear Drive Inverter Replacement ASSEMBLY - DRIVE INVERTER - 4DU LABEL - COVER - ACCESS - LUG - PHASE For Dual Motor Front Drive Inverter Replacement ASSEMBLY - DRIVE INVERTER - 4DU LABEL - COVER - ACCESS - LUG - PHASE For Dual Motor Rear Drive Unit Replacement REAR DRIVE UNIT - DUAL MOTOR CONFIGURATION | 1 2 2 1–2 1–2 1 1 1 1 1 |
| If Necessary | 1923000-00-C | INVERTER MOUNT | 1 |
|  | These part numbers were current at the time of publication. Use the revisions listed or later, unless otherwise specified in the Parts Catalog. |  |  |
| Special Tools | 1081765-00-A 1080568-00-A 1135762-00-A 1066521-00-A 1498673-00-A 1479505-00-A 1130279-00-A 1050448-00-B 1099645-00-D or 1099645-99-D 1130480-02-B 1056154-00-A 1026636-00-A 1479505-00-A 1080568-00-A 1080292-00-A 1450875-00-A 1065131-00-A 1135762-00-A 1050448-00-B 1081765-00-A 1080568-00-A 1135762-00-A 1066521-00-A 1942299-00-A 1498673-00-A 1054170-00-D 1479505-00-A 1065131-00-A 1050448-00-B 1130279-00-B or 1130279-99-B | For Tri-Motor Front Drive Unit Replacement 3/4 TON UNDERHOIST STAND OIL/COOLANT CATCHER KIT, SVC PLUG, COOLING HOSE, MODEL 3 POWERTRAIN POWERTRAIN LIFT KIT, CABLE AXLE REMOVER, MS/MX/M3 KIT, M3 VACUUM FILL COOLANT ADAPTER LIFTING SLING, DRIVE UNIT, MODEL 3 REFILLER, COOLING SYSTEM FIXTURE, FRONT SUBFRAME, MODEL 3/Y/CT For Drive Inverter(s) Replacement SLIM TEST PROBES, CAT III 1000V ANTI STATIC MAT /W WRIST GROUNDING CORD AND WRIST STRAP, 20" x 24" PACK ENCLOSURE LEAK TESTER, HV BATTERY KIT, M3 VACUUM FILL COOLANT ADAPTER OIL/COOLANT CATCHER GROUND STRAP, ESD WRIST STRAP INVERTER LEAK TEST KIT KIT, BATTERY COOLANT DRAIN & FILL KIT, SVC PLUG, COOLING HOSE, MODEL 3 REFILLER, COOLING SYSTEM For Dual Motor Rear Drive Unit Replacement 3/4 TON UNDERHOIST STAND OIL/COOLANT CATCHER KIT, SVC PLUG, COOLING HOSE, MODEL 3 POWERTRAIN POWERTRAIN LIFT CT REAR SUBFRAME FIXTURE KIT, CABLE AXLE REMOVER, MS/MX/M3 KIT, ALIGNMENT TORQUE TOOLS KIT, M3 VACUUM FILL COOLANT ADAPTER KIT, BATTERY COOLANT DRAIN & FILL REFILLER, COOLING SYSTEM LIFTING SLING, DRIVE UNIT, 3DU/4DU |  |
| Shop Supplies | Electrical Protective Gloves Brake Caliper S-Hook Plastic Trim tool |  |  |

# Procedure

- Double click the yellow box in the VIN lookup tool below, enter the full VIN, and then press Enter to determine which drive unit and/or drive inverter must be replaced.

- NOTE: Refer to the table below to define the VIN lookup tool output.

| - VIN Lookup Tool Output |  |
| --- | --- |
| - FDI | - Front Drive Inverter |
| - RDI | - Rear Drive Inverter (Dual Motor) |
| - FIM | - Front Inverter Mount |
| - RIM | - Rear Inverter Mount |
| - FDU | - Front Drive Unit |
| - RDU | - Rear Drive Unit |
| - LH RDI | - LH Rear Drive Inverter |
| - RH RDI | - RH Rear Drive Inverter |

- Refer to the following procedures to replace the affected drive unit and/or drive inverter as determined in Step 1. Use the Correction Description table to determine the correction code depending on which drive inverter and/or drive unit is replaced.

- NOTE: Some vehicles built before February 1, 2024 require an updated inverter mount. If the VIN lookup tool result includes an inverter mount replacement, replace the inverter mount as instructed. Correction code S132440009 for the inverter mount replacement will be automatically added to the Service Visit. Note that the time it takes to replace the mount is built into the FRT for the inverter replacement, therefore the FRT for correction code S132440009 is 0.00.

  - To replace the dual motor front drive inverter, refer to Service Manual procedure 3920048022.

  - To replace the dual motor rear drive inverter, refer to Service Manual procedure 4020092022.

  - To replace the dual motor front and rear drive inverters, refer to Service Manual procedure Front and Rear Drive Inverter - Set (Dual Motor) (Remove and Replace).

  - To replace the tri-motor LH or RH rear drive inverter, refer to Service Manual procedure 4020092032.

  - To replace the tri-motor LH and RH rear driver inverters, refer to Service Manual procedure Rear Drive Unit - Rear Drive Inverter - Set (Tri-Motor) (Remove and Replace).

  - To replace the tri-motor front drive unit, refer to Service Manual procedure 3901049112.

  - To replace the tri-motor front drive unit and LH rear drive inverter, refer to Service Manual procedure Front Drive Unit and Rear Drive Inverter - LH (Tri-Motor) (Remove and Replace).

  - To replace the tri-motor front drive unit and RH rear drive inverter, refer to Service Manual procedure Front Drive Unit and Rear Drive Inverter - RH (Tri-Motor) (Remove and Replace).

  - To replace the tri-motor front drive unit and LH and RH rear drive inverters, refer to Service Manual procedure Front Drive Unit Assembly and LH and RH Rear Drive Inverters (Tri-Motor) (Remove and Replace).

  - To replace the dual motor rear drive unit, refer to Service Manual procedure 4001093071.